| DELFT UNIVERSITY OF TECHNOLOGY Faculty of Electrical Engineering, Mathematics and Computer Science |  |
| --- | --- |

Final assignment
Microsystems Design and Modelling (ET4260)

Part 1/3

Instructions:

Document your calculations properly with derivations and screenshots when needed.

Please submit your files in word format to facilitate feedback. Latex-generated PDF files are also accepted but not preferred.

Deadline for first submission of Part 1: 13th May 2026

Deadline for final report submission: 24th June 2026

Please, mark corrections made after the first submission in blue.

________________________________________________________________

State here your NAME and STUDENT NUMBER.

________________________________________________________________

Optical phase shifter for quantum applications

Photonic integrated circuits are increasingly used in quantum information processing, where precise control of the optical phase is required to manipulate quantum states of light. One approach to achieve this control is through MEMS-based optical phase shifters, where mechanical motion modifies the effective refractive index experienced by light propagating in a waveguide.

The MEMS photonic phase shifter shown in the figure operates by coupling the evanescent field of light propagating through an optical waveguide to a silicon structure located at a small distance from the waveguide. By changing the gap g between the suspended silicon mass and the waveguide, the effective refractive index of the guided optical mode is modified, producing a phase shift in the transmitted light.

If the phase shift for the device at rest is denoted by ϕ0, a change in the gap g introduces an additional phase shift (unit in rad) given by

Δϕg=2πLλ neffg0-neffg

where λ=1550 nm is the wavelength of the light, L is the length of the modulation section, neffg is the effective refractive index across the modulation section, and g0 is the gap at rest (corresponding to Vd=0). The effective refractive index and the gap are related by the following exponential equation:

neffg=ηeff,0-ηeff,∞ e-bg+ηeff,∞

where b=0.3 μm-1 is a constant, ηeff,0=2.6 is the effective refractive index at g=0 and ηeff,∞=2.4 is the effective refractive index of the isolated waveguide (i.e., when g→∞).

The mechanical displacement of the suspended mass xm is controlled using an electrostatic comb-drive actuator, which produces a force on the suspended mass when a voltage is applied. The main geometrical dimensions and material properties of the device are summarized in the table on page 3.

Below is a real fabricated device as an example:

A SEM image of a fabricated device. Ref: Pierre Edinger, Alain Yuji Takabayashi, Carlos Errando-Herranz, Umar Khan, Hamed Sattari, Peter Verheyen, Wim Bogaerts, Niels Quack, and Kristinn B. Gylfason, "Silicon photonic microelectromechanical phase shifters for scalable programmable photonics," Opt. Lett. 46, 5671-5674 (2021)

*Some dimensions are not drawn to scale for better visualization.

| Length of modulation section | L | 100 μm |
| --- | --- | --- |
| Modulation gap at rest | g0 | 200 nm |
| Length of beams | Lb | 75 μm |
| Width of beams | Wb | 5 μm |
| Length of mass central section | Lm | 40 μm |
| Width of mass sections | Wm | 30 μm |
| Comb-drive gap | h | 1 μm |
| Comb-drive overlap length at rest | d0 | 19 μm |
| Comb-drive finger length | Lc | 20 μm |
| Comb-drive finger width | Wc | 1 μm |
| Thickness of suspended parts | t | 200 nm |
| Number of gaps in comb-drive | N | 40 |
| Young’s modulus of Si | ESi | 170 GPa |
| Density of Si | ρSi | 2330 kg/m3 |

Mechanical model and dynamics

Initially, assume the device operates in vacuum and neglect any damping effects.

Question 1) Write the lumped-element differential equation of motion for the suspended mass. Consider the electrostatic force generated by the comb-drive actuator  Fe as the system input and the displacement of the mass xm as the system output.

Question 2) Estimate the fundamental resonance frequency. For this calculation:

- Neglect the compliance of the central mass when estimating the stiffness of the system.

- Neglect the mass of the beams when estimating the effective mass of the system.

Question 3) Using the system constants estimated in Question 2, calculate the displacement of the suspended mass xm due to gravity, assuming that gravity acts along the direction of mass displacement.

In what follows, assume that the device operates in air. The surrounding air exerts a damping force Fd proportional to the velocity of the suspended mass xm and its top-side area Am according to the following expression. This simplified model neglects viscous damping contributions from the sidewalls of the structure.

Fd=cair⋅Am⋅xm

where cair= 16.15 N s/m3 is the air damping coefficient. Additionally, energy losses at the beam anchors can be accounted for using a quality factor Qclamping=500.

Question 4) Considering both damping mechanisms, calculate the settling time of the device for a step input force. The settling time is defined as the time it takes for the mass displacement to settle within ±5% of its final value.

Question 5) Mechanical vibrations can damage the device if the suspended mass comes into contact with the optical waveguide. These vibrations can be modelled as harmonic accelerations at a given frequency.

- Derive an expression for the maximum acceleration amplitude, as a function of frequency, that the device can withstand without the suspended mass touching the waveguide, assuming Vd=0. Assume the acceleration is applied in the direction of motion of the suspended mass.

- Using this expression, determine the maximum DC acceleration that the system can withstand.

- Using this expression, determine the maximum AC acceleration that the system can withstand at the worst-case frequency.

Electrostatic transduction and sensitivity

Question 6) Express the gap g as a function of a DC applied voltage Vd and plot the resulting phase shift Δϕ for a range of Vd from -50 V to 50 V. For this calculation, neglect fringing effects and consider only the electric field component perpendicular to the comb-drive fingers.

Question 7) Discuss the pull-in instability of the comb-drive actuator and, if applicable, derive an expression for the pull-in voltage under the following modelling assumptions:

- Consider only the electric field component perpendicular to the comb-drive fingers, neglecting fringing fields. (Meaning: consider only the capacitance generated by the overlapping area of the fingers).

- Include the field component along the direction of the comb-drive fingers. (Meaning: consider the parallel-plate capacitance between the finger tips and the opposing anchors, in addition to the overlapping capacitance).

To implement phase modulation at a frequency fm, the applied voltage is composed of a DC component and a small AC modulation component:

Vd(t)=VDC+VACsin2π fmt,        with VAC≪VDC

Question 8) Model the modulation response of the system by following these steps:

- Using a linearized small-signal model, derive the frequency response of the mass displacement xm with respect to the modulation voltage amplitude VAC, leaving the bias voltage VDC as a variable. For the comb-drive model, neglect fringing effects and consider only the electric field component perpendicular to the comb-drive fingers.

- Obtain the sensitivity of the phase shift Δϕ to the mass displacement xm at the bias point set by VDC, using a first-order Taylor expansion. Recall that the general first-order Taylor expansion for a function f(x) around an operating point x0 for a small change δx is:

fx0+δx≈fx0+dfdx​x0⋅δx

- Combine both results to obtain the small-signal transfer function of the system ΔϕsVACs. Sketch the corresponding Bode diagram using VDC=10Vand give expressions for the main performance metrics:

  - Low-frequency modulation sensitivity, or ΔϕsVAC(s) as fm→0.

  - Highest modulation sensitivity, or ΔϕsVACs at the device resonance frequency.

  - Slope of the roll-off.

- Plot the low-frequency modulation sensitivity as a function of VDC ovr the range from 0 to 5 kV and discuss the observed trends.

Geometric scaling

Question 9) Assume that all the dimensions in the design are multiplied by a factor of 1000. Discuss how the following main device characteristics scale with this change:

- System stiffness.

- System mass.

- Resonance frequency.

- Mass displacement due to gravity.

- Maximum AC acceleration that the system can withstand.

- Electromechanical coupling constant (η) of the comb-drive for a given bias voltage level, assuming the number of gaps in the comb-drive N stays unchanged.

---------- END OF PART 1 ----------